MitM Sniffing & SIEM Monitoring & Snort (NIDS)

# Data Sniffing Lab Report

## 1. Objective

The primary objective of this lab is to demonstrate how unencrypted network traffic (HTTP) can be intercepted using a Man-in-the-Middle (MitM) attack vector via ARP Spoofing, and to highlight the critical risk of transmitting sensitive data in plaintext.

## 2. Network Environment Setup

The lab environment consists of an isolated Host-Only network topology with the following components:

- Attacker Machine: Kali Linux VM (192.168.56.105)

- Target Machine (Host): Windows Laptop (192.168.56.1)

- Vulnerable Web Server: Metasploitable 2 VM (192.168.56.106)

## 3. Step-by-Step Execution Procedure

### Step 1: Network Reconnaissance

The network interface  eth1 was initialized inside  Bettercap  on the attacker machine using the command:

A network probe was executed using  net.probe on to discover active hosts. The net.show command successfully mapped the active IP addresses and MAC mappings of the environment.

### Step 2: Launching the ARP Spoofing Attack

Due to the absence of an external virtual gateway on the Host-Only network adapter, a targeted internal ARP spoof was initiated. The target was specified to direct traffic securely through the attacker's instance:

This forces the target host machine to update its local cache, positioning the Kali Linux machine directly in the middle of the communication stream.

### Step 3: Traffic Sniffing Activation

To observe the plaintext payloads in real-time, the packet sniffer module was enabled:

Plaintext

on

## 4. Analysis and Proof of Concept (Credential Capture)

The target initiated an unencrypted HTTP session by navigating to the Damn Vulnerable Web Application (DVWA) login interface hosted at http://192.168.56.106/dvwa/login.php.

Once the victim submitted their credentials on the application interface, the packet sniffing module captured the outbound POST request parameters traversing the local interface.

### Intercepted Evidence Payload:

HTTP

Observations: As demonstrated in the console output, Bettercap successfully isolated the exact parameter strings for both the username and password fields in clear text without requiring decryption mechanisms.

## 5. Defensive Mitigations

- Enforce HTTPS (TLS/SSL): All web applications must use encrypted channels to prevent plaintext data from being readable during transport layer sniffing.

- Static ARP Configuration / DAI: Implementing Dynamic ARP Inspection (DAI) on network switches prevents unauthorized devices from broadcasting rogue ARP replies.

- Network Monitoring (IDS): Utilizing intrusion detection systems like Snort configured on the interface can immediately flag repetitive or inconsistent ARP mapping patterns.

6. Snort Alert Generation:

Bhai, report ko bilkul short aur to-the-point kar diya hai. Ise copy-paste karke directly apne Word document ya Google Docs mein daal lein, yeh minimum pages gheregi:

## Wazuh Monitoring:

## 1. Objective

To execute an ARP Spoofing Man-in-the-Middle (MitM) attack to capture cleartext credentials and verify centralized event logs using the Wazuh SIEM Dashboard.

## 2. Setup Details

- Attacker: Kali Linux VM (192.168.56.105)

- Victim: Windows Laptop Host (192.168.56.1)

- Target Server: Metasploitable 2 (192.168.56.106)

- SIEM Platform: Wazuh Appliance (v4.14.5)